Check_Network_MemoryIncident Management

1: Check the memory of the device, if really free memory available, please check with solarwinds team for threshold level set.

1: To see memory need to use command:

show version

2: show memory : this command will show the total memory, used, free largest and lowest memory.

3: show processes memory: This will present active processes and memory used by each process.

4: show processes memory platform sorted: This command will show system memory utilization for individual processing running

5: monitor platform software process rp active: This command to monitor system memory usage for each process running

6: If alert is genuine, if memory is high, then need to check memory utilized by IoS platform and need to raise TAC case with vendor.

7: Recommendations need to implement with CRQ.